6 Zones Mixer Amplifier with Mp3

TI-1206S/TI-2406S/TI-3506S/TI-5006S

Features  TI-1206S/TI-2406S/TI-3506S/TI-5006S

* 4 Channel Microphone and 2 line input, treble and bass can be controlled separately .

* 6 Channel Partition output, and partition volume independent control.

* Built in bell. Bell tone optional: 4 tones and 2 tones (built-in), alarm generator.

* Mp3 decoding module supports usb/sd/bluetooth/fm radio playback function. With

the MP3 remote control, support power off memory function, MIC recording function.

* When the alarm generator starts, background music will be closed and6 partition

output will automatically open to maximum output volume.

* Built in 24V DC power input.

* Mic1- 4 built-in 48V phantom power supply, and switch can be independently

controlled.

* T-216 partition control microphone can remotely control partition output, and it can

be cascaded up to 6 units.

* Audio input with independent control on tone control and total volume control.

* A variety of indicator lights (signal, the top of the roof, to protect the LED indicator)

*  Using fan forcible cooling structure, allowing the machine to work for a long time.

* There are 5 priority functions:

The first level EMC priority: when input the EMC alarm signal, all the sound source will

be turn off, and the 6 Channel partition output will automatically open.

Second level:  tel/rem mic override mic1-4, line1-2 and bell tone.

Third level: built-in alarm generator override bell sound, mic1-4, line1-2, 6 partition

output will automatically open.

Fourth level: built-in bell tone override mic1-4, line1-2.

Fifth level: built-in mic1 override mic2-4, line1-2.

* Constant voltage output: 70V, 100V, rated resistance output: 4-16 Ω.

Specifications

| Model |  | TI-1206S |  | TI-2406S |  | TI-3506S |  | TI-5006S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Amplifier Section (Load 100V Output) | Rated Output Power (THD 1%) | 120W(RMS) |  | 240W(RMS) |  | 350W(RMS) |  | 500W(RMS) |
|  | Speaker Zone 1~5 Output | 25W MAX |  | 50W MAX |  | 70W MAX |  | 85W MAX |
| Mic Input Section (Mic 1~4 In@1KHz) | Input Sensitivity/Impedance | 5mV 600Ω/350mV 10KΩ BAL |  |  |  |  |  |  |
|  | S/N Ratio | Better than 75dB |  |  |  |  |  |  |
|  | Frequency Response (±3dB) | 80Hz~16KHz |  |  |  |  |  |  |
|  | Tone Control (Bass:100Hz, Treble:10KHz) | ±10dB |  |  |  |  |  |  |
|  | Phantom Power | + 48V |  |  |  |  |  |  |
| Line Input Section (Mic 1~2 In@1KHz) | Input Sensitivity/Impedance | 350mV/47KΩ |  |  |  |  |  |  |
|  | S/N | Better than 80dB |  |  |  |  |  |  |
|  | Frequency Response (±3dB) | 80Hz~16KHz |  |  |  |  |  |  |
|  | Tone Control (Bass: 100Hz,Treble:10KHz) | ±10dB |  |  |  |  |  |  |
| Tel Audio Input | Input Sensitivity/Impedance | 0.775V/10KΩ |  |  |  |  |  |  |
|  | S/N | Better than 75dB |  |  |  |  |  |  |
|  | Frequency Response (±3dB) | 50Hz~16KHz |  |  |  |  |  |  |
| EMC Audio Input | Input Sensitivity/Impedance | 0.775V/10KΩ |  |  |  |  |  |  |
|  | S/N | Better than 75dB |  |  |  |  |  |  |
|  | Frequency Response (±3dB) | 80Hz~16KHz |  |  |  |  |  |  |
| Pre Amp Out(Mic 1~4, Line 1~2, Tel In, Remote In) |  | 1V/600Ω |  |  |  |  |  |  |
| Remote MIC Sensitivity/Impedance |  | 775mV/10KΩ |  |  |  |  |  |  |
| THD |  | Less than 0.1% at 1KHz, 1/3 rated power |  |  |  |  |  |  |
| Power Supply |  | ~110-230V 50/60Hz & DC 24V |  |  |  |  |  |  |
| Power Consumption |  | 180W | 320W |  | 500W |  | 750W |  |
| Weight |  | 6.5Kg | 7.1Kg |  | 7.8Kg |  | 8.9Kg |  |
| Dimensions |  | 484x310x88 mm |  |  |  |  |  |  |

Rear Panel   TI-1206S/TI-2406S/TI-3506S/TI-5006S